Using SoftPLC NT Realtime

In this document it is assumed that the reader is familiar with the fundamental behaviour and functions of a CoDeSys runtime system. Therefore this brief manual only deals with the specific features of the runtime system SoftPLC NT Realtime. This is a real time extension (RTE) of Windows NT.

Please prepare the SoftPLC NT Realtime by calling the adequate program in the CoDeSys directory. In the Windows task bar the service icon appears        .

# What does real time extension mean?

A real time system is characterised by a predictable (deterministic) time behaviour. If for example a real time system is given the task of executing specific routines (through configuration) within a predetermined time pattern this will happen within predetermined temporal ranges of tolerance. If this is not the case it is regarded as a failure of the whole system.

If this is applied to a controller such as CoDeSys this means that a task is called within predetermined (known) ranges of tolerance.

Figures drawn from past experience have shown that these ranges are not adhered to by Windows NT, which means that NT is not a genuine real time operating system.

Yet the hardware in a PC is made in such a way that it is possible to create a reliable task scheduling by means of software.

The real time extension of Windows NT is an NT driver which installs an Interrupt-Service-Routine which is called-up cyclically by the timertick of the PC’s hardware.

This routine now takes over the job of calling-up defined tasks from CoDeSys and/or of authorising the continued or renewed execution of the operating system.

## The System in details

# Operation via the Service Icon

Bellow the context menu (right mouse button) of the RTE Icons in the task bar:

The operations which are sensible to conduct are presented in black (enable) depending on the current operational condition.

The following individual operations can be conducted:

- Start System loads and activates the driver. Only then is it possible to work on real time tasks and communicate with the programming system.

- Stop System stops the driver and removes it from the PC’s memory.

- Start Cycle / Stop Cycle  starts and stops the cyclical execution of IEC tasks. This operation corresponds with the menu item Start/Stop in the online menu of CoDeSys.

- Next are the three types of reset options which also correspond with the entries in the CoDeSys menu.

- About shows the splash screen of the RTE which apart from the icon also includes information on the version and the copyright.

- System shows a dialogue with system settings. Two settings can be altered here: If there is a tick behind the option „Start when booting“ the service is started automatically by NT on the next start of the system. This happens completely regardless of the fact whether a user logs in or not. With the option „Automatic PLC Start“ the user can decide whether to start the driver automatically from the service or not.

- Exit not only stops the driver but also the service after the driver has been unloaded. The service can be restarted with „Start – Settings– System Control – Services“ or by the entry in „Start – Programs“ under CoDeSys. It is possible to start the driver in „System Control – Devices”, yet this is not very sensible because it deactivates the driver. The RTE service activates it when starting.

Note: When the driver is activated by the service at the start an existing boot project is loaded. It thus becomes possible to start the system automatically and also start a boot project without a user being logged into the PC by simply activating the two options in the system dialogue.

More information on IO drivers can be found in the RTIODrv-Toolkit and the document that describes the interface to runtime systems.

Mentioned above are only the registry entries which the run time system uses directly. There may however be more entries that are used by the complete system because the I/O drivers naturally set up and administer their own necessary entries. As before for example the I/O driver RTIOdrvHilscherDPM uses the data in RegKeyRuntime\IO Devices\.0 to 4 to find its cards.

# Configurable Features

In this context a configurable feature is a feature of the run time system which is dependant on parameters which can be altered by the user without having to rebuild the run time system. At the moment these possibilities of configuration are restricted to entries into the registry of Windows NT.

Yet in the broader sense an external IO driver with a name that belongs to the configurable parameters can also implement such features.

## The registry entries applied by the run time system

The key in the registry called  RegKeyRuntime here has the following path:

\\HKEY_LOCAL_MACHINE\SOFTWARE\3S-Smart Software Solutions GmbH\CoDeSys SP\RTPLC

The run time system searches for the following data under this key:

| Name | Possible entry | Meaning |
| --- | --- | --- |
| LastExit, DWORD | 0 or 1 | Before the attempt is made to load a boot project LastExit is set 0. If the PLC breaks down (is not shut down normally), this setting can no longer become 1.No further boot project is loaded if the setting is 0. |
| NmiConnect, DWORD | 0 or 1. | If this entry is made and equals 1, an interrupt handler of the run time system is placed on the Int-vector 2. at the moment only sensible together with NmiSchedule. |
| NmiSchedule, DWORD | Bitwise usage. | Bit0: If this bit is set, the time interval is not changed and no specific routine is place on the interrupt vector of the timer chip. The Scheduler is called up by the Nmi-Handler routine. Bit1: If set, the IO-driver that handles the NMI determines, when NT must be scheduled. Bit2: If set, the RTE-kernel will not install an interrupthandling routine for the cyclic schedulerinterrupt. A suitable IOriver must do this. The IOdriver has to be able to generate an interrupt on demand and call the NMIhandlerroutine of the runtimekernel. |
| MaxNonNtInt, DWORD | 50 – 950 | If the scheduler works on a Nmi , which is generated asymmetrically (with the goal of giving NT less CPU time than the PLC) this entry determines after which interrupt and after which IEC task NT is activated. The IEC task is always the decisive factor. |
| MaxPLCTime, DWORD | 10 – 90 | If scheduling is being done conventionally i.e. with the timer tick this figure determines how much time NT is given compared to the time given to the run time system. This means that if the figure specified here is 90 the PLC is given 90% of the CPU time while NT is only given 10%. |
| Path, STRING | \??\<valid directory name that starts with the letter of the drive> | The path under which all files which affect the run time system are be searched for and saved. Maximum 128 characters. |
| RetainFlags, DWORD | all 32 bits are used individually | Default: Retain memory is allocated dynamically in the RAM ,save in file when ending. Bit0 = 1: Retain memory is not allocated dynamically. Bit1 = 1: Retain memory is placed on physical address. If only bit 0 and bit 1 is set (3) it is assumed that the retain sector is located directly in an SRAM. The retain data is not saved explicitly. Bit2 = 1: Save retains in file when ending, like default. Bit3 = 1: Save retains using IO driver, cyclical. |
| RetainAddr | Valid physical storage address | Must be available if Bit 1is set in retain flags. This entry is used as the physical storage address for saving the retain data. The length is specified in RetainSize. |
| RetainSize | Size of all the retain data (under the physical address or how many are to be allocated) | Must be available if Bit 1is set in retain flags. |
| HandleBlueScreen | 1 or 0 | If this entry is available and 0 the NT-Bluescreen is generated when the system breaks down. If this entry is not available or 0 the computer „freezes“ in case of a break down. The PLC continues to operate (if it did not cause the break down itself). |
| StopIODrivers | 0 or 1 | The RT service starts all the listed IO drivers automatically. If this setting is 0 it also stops them automatically when ending the system. |
| OutputFlags | all 32 bits are used individually | Bit 0: 0: The outputs are not forced, when PLCstate changes to stop. 1: The Outputs are all forced to the value of Bit1 (in this entry), when PLC changes to STOP. |
| StackpoolSize | Size of the pool in bytes. Too big will decrease system performance, too small will lead to stack overflow of an external task (may crash the system). | The size of the memorypool, where the extern (by IODrivers) created tasks take their stack from, is defined here. The pool is allocated when starting the system. When specifying a value smaller than 512bytes, the IODrivers are not able to create their own tasks. When nothing is specified, the default is 64k. |
| CodeSize | Size of the codearea in bytes. | The PLC allocates 2 of these areas to store the code generated by CoDeSys. The default is 4MB, if the entry is missing. |
| DataSize | Größe des Datenbereichs in Byte | The PLC allocates of this aea to store the data generated by CoDeSys (IEC-variables, memoryarea and PI). The default is 2MB, if the entry is missing. |
| WdgMultiplier | 0 – 0xfffffffe , 0xffffffff : disable | The number of cycletimes, a task can last longer than specified in the project, before the watchdog will stop the PLC. With a value of 0 (default) the watchdog will stop the system at once, if a task violatest its cycletime. A value of 0xffffffff will disable the watchdog. |
| Resolution | 50,100,200 | The timeresolution in µs the scheduler is called. (Only for PLCs with the target-id 44 or 45) |
| InterruptFlags | 0 or 1 (at the moment) | If 1, interruptfunctions are called in an environment where FPU-operations are allowed. |

There is a list of IO driver names under the subkey of RegKeyRuntime „IO Drivers“. The name of the driver can have two forms: <drivername> or <driverobjectname><space><drivername>. The second form for the name is required, if the driver creates a driverobject, which is named different from the driver itself. The entries have to be called driver<lfd.number>. The Ifd number can be anything from 0 to 4. The entries are sequences of characters. The drivers are called using the normal mechanisms (e.g. CreateFile) with these sequences.

More information on IO drivers can be found in the RTIODrv toolkit and in the document which describes the interface to run time systems.

There may exist a subkey of RegKeyRuntime „External  Dlls“, where a list of DLLs, loaded when system starts, is located.The list is of the form:

Dllx		<optional path><filename>

x is a number, counting from 0. The functions in the Dlls can be called by using a functionblock from the „DllCall“-library.

Mentioned above are only the registry entries which the run time system uses directly. There may however be more entries that are used by the complete system because the IO drivers naturally set up and administer their own necessary entries. As before for example the IO driver RTIOdrvHilscherDPM uses the data in RegKeyRuntime\IO Devices\.0 to 4 to find its cards.

# Systemdiagnostic

## Busdiagnostic of IOdrivers

All IOdrivers are called to fill a diagnostic structure, each time an IECtask completed its IO-update. TYPE GETBUSSTATE:

STRUCT

BOLDENABLE : BOOL;

ENABLE: BOOL;

DRIVERNAME:POINTER TO STRING;

DEVICENUMBER:INT;

READY:BYTE;

STATE:INT;

EXTENDEDINFO:ARRAY[0..129] OF BYTE;

END_STRUCT

END_TYPE

The structure is defined in the library BusDiag.lib.

The IO-drivers write to the address specified for each busmaster in CoDeSys configuration. The function is only active for masters which are configured with CoDeSys.

For each busmember a byte in EXTENDEDINFO is reserved, where the first 3 bits are used:

Bit 0: Busmember exists in configuration.

Bit 1: Busmember is available on the bus.

Bit 2: Busmember signalizes an errorstate.

If a busmember is signalizing an errorstate, the specific diagnosticdata can be retrieved by using the functionblock DiagGetState (also defined in BusDiag.lib).

The FB DiagGetState is called for a specific busmember.

FUNCTION_BLOCK DiagGetState

VAR_INPUT

ENABLE:BOOL ;

DRIVERNAME:POINTER TO STRING ;

DEVICENUMBER:INT ;

BUSMEMBERID:DWORD ;

END_VAR

VAR_OUTPUT

READY: BOOL ;

STATE:INT ;

EXTENDEDINFO:ARRAY[0..99] OF BYTE ;

END_VAR

The FB starts its work with a rising edge on its ENABLE-input.

DRIVERNAME is the name of the IOdriver, which is responsible for the master of the busmember. If 0 is specified here, the diagnosisjob is deliverred to all eexisting drivers.

DEVICENUMBER identifies the bus, managed by the driver. (For example, the Hilschercarddriver can manage up to 5 cards (busses).) The index is 0-based.

BUSMEMBERID identifies the busmember. When using a CANopen-card, the BUSMEMBERID is the nodeID, using a PB-card the stationadress is used to identify the busmember

READY is set by the FB, to signalize, the job has been done.Then the STATE-member is valid and contains a value from

VAR_GLOBAL CONSTANT

NDSTATE_INVALID_INPUTPARAM:INT:=-1;

NDSTATE_NOTENABLED:INT:=0;

NDSTATE_GETDIAG_INFO:INT:=1;

NDSTATE_DIAGINFO_AVAILABLE:INT:=2;

NDSTATE_DIAGINFO_NOTAVAILABLE:INT:=3;

END_VAR

In the bytearray EXTENDEDINFO up to 100 bytes of manufacturer-specific diagnosticdata of the busmember are available now.

## „System diagnostic“ in the „System...“-menu

Within the servicemenu you can get through „System ...“ to the dialog with some systemsettings. Here exists the button  „System diagnostics“.

Some essential settings and errormessages can be read here.

The currently used IO-drivers, some information about their devices and initialization- and runtimeerrors are displayed.

The progressbar „Current PLC-load“ shows the current usage of the PLC, in percent. Remember: The PLC uses the  CPU of the PC max. „MaxPLCTime“-Percent (see the description of the registry entries above. This timeslice is 100% here.)